MATERIAL SAFETY DATA SHEET

Crop Smart LLC	 						In Case of Emergency, Call:

P.O. Box 342								24 Hour Emergency Number

Morrisville, NC 27560						CHEMTREC: 800-424-9300

Medical Emergency Contact:

1-800-308-1241

1. CHEMICAL IDENTIFICATION

Product Name:		CropSmart Glyphosate 41 Plus					Product No.:  000121

EPA Signal Word          Caution

EPA Reg. Number:	42750-61-72693

Active Ingredient (%)	Glyphosate, isopropylamine salt  (41.0 %)

Chemical Name:	Glyphosate: Glycine, N-(phosphonomethyl)-compd. with 2-porpanamine (1:1)

Chemical Class:		Herbicide

2. COMPOSITION/INFORMATION ON INGREDIENTS

OSHA	      ACGIH			         NTP/IARC/OSHA

# Material			    PEL		      TLV		Other	         Carcinogen_________

Glyphosate Isopropylamine Salt	     ND		        ND				No

(38641-94-0)

Ethoxylated Tallowamines	     ND                       ND				No

(61791-26-2)

3. HAZARDS IDENTIFICATION

EMERGENCY OVERVIEW:  Hazardous chemical.  Clear amber liquid. Odorless to slight amine-like odor. Do not mix store or apply this product or spray solutions of this product in galvanized or unlined steel (except stainless steel) containers or spray tanks. May cause severe but temporary eye irritation.   Emergency personnel should wear a self contained breathing apparatus because noxious fumes are produced under fire conditions.

Potential Health Effects:  This section includes possible adverse effects which could occur if this material is not handled in the recommended manner.

Eye:              May cause severe irritation with redness and tearing.

Skin:             A single dose exposure to the material is slightly toxic and irritating.

Ingestion:     Single dose oral toxicity is low.  Amounts ingested incidental to normal handling conditions are not

likely to cause injury.  Ingestion may cause gastrointestinal discomfort with irritation of the mouth,    nausea, vomiting and diarrhea. Oral ingestion of large amounts may result in hypotension and lung edema.

Inhalation:  Single exposure to vapors is not likely to be hazardous.  Avoid breathing vapors or mist.

Systemic (other Target Organs) Effects: Signs and symptoms of excessive exposure may be nausea and/or vomiting  and abdominal cramps and/or diarrhea.  Birth defects are unlikely to occur.

## Physical Properties

Appearance:  Clear amber liquid.

Odor:  	          Odorless to slight amine-like odor.

4.   FIRST AID MEASURES

Ingestion:	     May cause gastrointestinal tract irritation.  Give a large glass of  milk or water and contact a

physician or poison control center immediately.

Eye Contact:	          Flush eyes with water immediately and continuously for 15 minutes. Consult a physician.

Skin Contact:	      Wash skin with plenty of soap and water for 15 minutes.  Remove contaminated clothing and

wash before reuse.  Consult a physician if irritation develops.

Inhalation:	     Remove to fresh air.  Consult a physician if irritation develops of breathing is difficult.

5.   FIRE FIGHTING MEASURES

#### Fire and Explosion

Flash Point (Test Method):	> 113oC (Pensky Martens closed tester)

Flammable Limits (% in Air):

Autoignition Temperature:         ND

Flammability:		             ND

Hazardous Combustion Products:       Noxious fumes produced under fire conditions may include carbon

monoxide, carbon dioxide, phosphorus pentoxide, and nitrogen oxide.

Extinguishing Media:                       Water spray , foam, fog, or carbon dioxide.  Do not allow runoff to enter

sewers or waterways

Fire Fighting Instructions:               Prevent human exposure to fire,  fumes, and smoke. Avoid heavy hose

streams and dike area to prevent runoff.

Protective Equipment for Fire Fighters:  Emergency personnel should wear full face, self contained breathing

apparatus and impervious protective clothing

6.   ACCIDENTAL RELEASE MEASURES

## In Case of  Spill or Leak  Contain spill.  Absorb spill with sand, floor clay or dirt.  Shovel or sweep up and mop

## affected area with soap and water. Prevent liquid from entering sewers and waterways.

## Large spills that soak into the ground should be dug up, placed in drums and disposed of

## properly.

7.   HANDLING AND STORAGE

Precautions to be Taken in Handling and Storage:   Good housekeeping  is necessary.  No smoking, open flames or sources of ignition in handling and storage area.  Wash thoroughly with soap and water after handling.  Do not contaminate water, food or feed by storage or disposal..  Do not store near seeds, fertilizers, insecticides or fungicides. Spray solutions of the product should be mixed, stored, and applied using only stainless steel, aluminum, fiberglass, plastic or plastic-lined containers.

8.   EXPOSURE CONTROLS/PERSONAL PROTECTION

These precautions are suggested for conditions with a high potential for exposure.  If handling procedures are such that there is only a low potential for exposure, less protection may be needed.  Emergency conditions may require additional precautions.

Engineering Controls:    Normal room ventilator should be satisfactory.  Eyewash stations and safety showers

should be easily accessible.

Eye Contact:	        Use chemical goggles.  Eye wash station should be located in the immediate work area.

Skin Contact:	        Use protective clothing impervious to this material.

Inhalation:	        Use a NIOSH approved respirator when protection is required.

9.   PHYSICAL AND CHEMICAL PROPERTIES

Appearance:                          Clear,amber liquid

Odor:	                            Odorless to slight amine-like odor.

Flash Point:       	        >1130C,  >2350F (Pensky Martens closed tester)

Specific Gravity/Density:      1.16 / 9.75

pH:	                             4.5 (1% solution in water)

10.   STABILITY AND REACTIVITY

### Reactivity

Stability:        		Stable

Hazardous Polymerization:         Will not occur

Conditions to Avoid:	             Do not mix, store, or apply this product or stray solutions of this product in

galvanized or unlined steel containers or spray tanks. This product could react with these containers to produce hydrogen gas which may form a highly combustible gas mixture.	         Caustic (basic) materials; galvanized or unlined steel containers.

### Hazardous Decomposition Products     Phosphorus pentoxide and nitrogen oxides under fire conditions.

11.   TOXICOLOGICAL INFORMATION

### Acute Toxicity/Irritation Studies

Ingestion:       LD50  (rats)        >  5000 mg/kg

Dermal:          LLD50  (rabbit)  > 2000 mg/kg

Inhalation:      LC50 (rat 4 hr.)  > 4.86 mg/l

Eye irritation  (rabbit 24 hr)      = slight to moderate irritation.

Mutagenicity: Mutagenic effects are unlikely

Teratology (Birth Defects):       Birth defects are unlikely.  Exposures having no effect on the mother should

have no effect on the fetus.  Did not cause birth defects in animals.  Other

effects were seen in the fetus only at doses which caused toxic effects to the

mother.

Carcinogenic Potential:           EPA has classified glyphosate in category E (evidence of non-carcinogenicity

for humans).

12.   ECOLOGICAL INFORMATION

### Glyphasate has been shown to be slightly to moderately toxic in aquatic studies and has been shown to be practically non-toxic to avian species following subacute dietary exposure.

Mallard duck		LC50		>2000 mg/kg

Bobwhite quail		LC50		>2000 mg/kg

Rainbow trout		96-hrLC50	18.6 mg/l (static)

Bluegill Sunfish	 	96-hrLC50	11.9 mg/l (static)

Algae			72-hrEC50	17.4 mg/l

13.   DISPOSAL CONSIDERATION

### Disposal Method:

14.   TRANSPORT INFORMATION

### B/L Freight Classification:    Weed killing compound,  N.O.I.B.N

15.   REGULATORY INFORMATION

### NOTICE:  The information herein is presented in good faith and believed to be accurate as of the issue or revision date shown below.  However, no warranty, express or implied, is given.  Regulatory requirements are subject to change and may differ from one location to another, it is users responsibility to ensure that its activities comply with federal, state and local laws and regulations.

### SARA Title III Classification

Section 311/312:   		Hazard Classes:   Immediate.

Section 313 chemical(s):	NA

### TSCA Status:  All components are on the US EPA’s TSCA inventory list.

16.   OTHER INFORMATION

NFPA Hazard Ratings

- Least

- Slight

- Moderate

- High

- Severe

Health:		1

Flammability:	0

Reactivity:	0

Issued Date: 	December 18, 2000

Supersedes: 	December 18, 2000

Reference MSDS : Cheminova, Glyfos X-TRA, dated April 2000.

The information herein is presented in good faith and believed to be accurate as of the issue or revised date shown above.  However, since the actual use of the product is beyond manufacturers control, no warranty, expressed or implied, is given and no responsibility assumed.  Regulatory requirements governing this product are subject to change and may differ from one location to another.  It is the buyer’s responsibility to ensure that the buyer’s activities comply with federal, state and local laws.